MBEYA UNIVERSITY OF SCIENCE AND TECHNOLOGY

DEPARTMENT OF APPLIED SCIENCES

PRACTICAL MANUAL

FOOD CHEMISTRY

Lecturer:

Laboratory Technician:

TABLE OF CONTENTS

PRACTICAL PROGRAMME	ii

EMERGENCY TELEPHONE NUMBERS	iii

LABORATORY SAFETY AND HEALTH RULES	1

PRACTICAL 1	6

LABORATORY ORIENTATION	6

PRACTICAL 2 QUANTITATIVE ANALYSIS OF FOOD COMPONENTS	7

2.1: DETERMINATION OF CRUDE PROTEIN BY KJELDAHL METHOD	7

2.2: DETERMINATION OF TOTAL FAT BY MANUAL EXTRACTION	12

2.3: DETERMINATION OF STARCH BY ACID HYDROLYSIS METHOD	15

2.4: DETERMINATION OF INDIVIDUAL SUGARS BY HIGH PERFORMANCE LIQUID CHROMATOGRAPHY (HPLC)	17

2.5: DETERMINATION OF ASH BY GRAVIMETRIC METHOD	23

PRACTICAL 3	26

DETERMINATION OF CHEMICAL PROPERTIES OF FOODS	26

3.1: DETERMINATION OF pH and TITRATABLE ACIDITY	26

3.2: DETERMINATION OF ASCORBIC ACID OR VITAMIN C	29

3.3: DETERMINATION OF TOTAL SOLUBLE SOLIDS	31

PRACTICAL REPORT GUIDELINES	34

FORMAT OF REFERENCES IN REFERENCES LIST	41

PRACTICAL REPORT MARKING SCHEME	47

# PRACTICAL PROGRAMME

| DATE | TIME | CONTENT |
| --- | --- | --- |

# EMERGENCY TELEPHONE NUMBERS

| Institution | Telephone Numbers |
| --- | --- |
| University Security Office |  |
| Police (emergency) |  |
| Police charge office/enquires |  |
| Fire department |  |
| Ambulance |  |
| Student Health Services |  |
| Hospital |  |
| Useful numbers |  |
| Departmental Risk Committee |  |
| Trained First Aid Staff |  |

# LABORATORY SAFETY AND HEALTH RULES

- General

- Conduct all assignments and requests from the lecturer and/or technician promptly according to the above-mentioned rules.  Choosing shortcuts can be perilous.

- Open shoes, sandals, shorts, short skirts, ties and excessive clothing and long hair that is not tied u, can be dangerous and cause accidents and are thus not allowed.  Long hair must please be tied up.

- Walking around the building barefoot is prohibited

- Wearing protective clothing (clean white coats), safety glasses and equipment are compulsory when prescribed.

- Smoking, eating or drinking are not allowed in laboratories.  Never store food in a refrigerator in which chemicals are stored.

- Do no sit or lie on laboratory work surfaces.

- Work intelligently and independently of other students as far as possible.

- Clean and neat laboratory practices are required as far as possible.  Leave your work area and the rest of the laboratory clean and tidy.

- Bunsen burners must always be placed on asbestos slabs when used, to protect the work surfaces.

- Playing or horsing about in the laboratory is strictly prohibited.

- If something is unclear, please ask for more information.

- Handle apparatus in a professional manner and if you are not a hundred percent sure as to how the apparatus works, rather ask.  Use balances carefully, clean after use and close the windows.

- Report all broken apparatus immediately to the supervisor or technician.

- Report any maintenance problems to the laboratory in charge or supervisor

- Remember to fill out logbooks of apparatus.

- Do not ever slam or kick doors of incubators, ovens, freezers or fridges.

- Handling Chemicals

- All chemical solutions MUST be marked with the following information: type of chemical; concentration; date prepared; and student’s name (name and surname).

- NEVER put chemicals in unmarked containers.

- Never sniff chemicals directly.

- Sucking up liquids with a pipette using the mouth can be detrimental.  Always use your pipette filler.

- Contact or accidental inhalation of dangerous substances must be reported to the supervisor immediately.

- Immediately rinse splashes from the skin using plenty of cold water; use polyethylene glycol to rinse off lipophilic substances.  Never use organic substances. Since may be absorbed

- Corrosive substances entering the eye should be thoroughly rinsed out with a gentle of water (or using special eye-wash equipment). Hold the eyelids wide open and move the eyes in all directions.  Consult an eye specialist immediately, stating the chemical substances involved.

- Immediately remove any clothing that has become contaminated with a corrosive substance.

- Be sure to seek medical advice in the event of an accident or if you feel unwell.

- Proper experimental planning is necessary, especially when working with flammable, poisonous or explosive substances.

- Do not experiment with chemicals.  Tasting of chemicals is also prohibited.

- Fume cupboards must be used-especially if vapour of gasses that may be present, are unknown.

- When bottles containing dangerous substances are opened, wear eye protection and gloves,

- Do not carry chemical bottles by the lids.  Use a special basket to carry chemical bottles

- Do not pour reagents back into bottles it will cause contamination

- Wipe any substances that have spilled immediately.  Do not leave teardrops on bottles and do not switch pipettes between bottles.

- Wash all glassware thoroughly using soap and water. If glassware does not want to come clean, ask the supervisor for help.

- All apparatus must be left clean and your hands must be washed thoroughly if dangerous chemicals have been handled.

- Before using chemicals, first read the label and check for WARNINGS.  Familiarize yourself with the warnings and take the necessary precautions see the list of warning codes attached and look at the poster (in the upstairs hallway) explaining the safety symbols.  When ordering new chemicals, get the Material Safety Data Sheet (MSDS) from the supplier.

- Accidents and Injuries

When you (or someone else) are involved in an accident or are injured, remember the following:

- Remain calm

- If an acid or alkaline substance is split on the injured, wash that area under the tap or emergency shower.

- If it is a cut or any other wound, call the supervisor and /or first aid person to help.

- Know where the nearest basin, emergency shower, eyewash bottle and the first aid box are situated.

- Fire

- Follow the rules as for gas flames

- Make sure that the gas burner taps are closed properly after use.

- Know where the closest fire extinguisher is and how to use it

- If possible, close the main gas valve (marked in yellow) in your laboratory, close all windows and close the door when everyone in the laboratory is out.

- Emergency Conditions

- Call the supervisor and emergency helpers

- Know where the closet telephone is and phone the US protection unit to report the incident.

- Give basic details so that they can properly inform the emergency services

- Familiarize yourself with the emergency plan of the building, the evacuation procedures and the meeting-place.

- Biological Safety

- All biological materials have to be autoclaved – red bins are available for petri dishes. If the bins are full, clean autoclave bags should be taken from the person in charge or laboratory supervisor. The bags are kept in the fridge until ¾ full, before being autoclaved

- All tips (yellow and blue) must be autoclaved and may not be placed in regular waste bins under any circumstances.  Anyone who breaks this policy will be fined.  Special containers will be available in all laboratories and in the walk-in fridge for tips and sharps (eg needles).

- All glassware used for sludge, or used to grow bacteria, must be autoclaved before washing.

- When working with sludge, pathogenic bacteria or hazardous chemicals is compulsory to wear white coats, gloves, masks and goggles.

- You must work in fume cupboards when working with sludge, effluent or hazardous chemicals.

- Broken Glass

- Never put broken glass into the regular bin. Use the green or blue bins provided in each laboratory.

- Always clean glassware directly after use.  Residue can harden and ay become difficult to remove especially in bottles.

- Cleaning of Laboratories

There are cleaners responsible for cleaning laboratory floors and garbage bins and the students are responsible for cleaning work surfaces and the rest of the laboratory.  Students are responsible for cleaning everything they worked with.

# PRACTICAL 1 # LABORATORY ORIENTATION

Objectives

- Discussion of laboratory safety rules

- A brief introduction to working in the laboratory

- Practical grouping of students

- Equip group cupboards with the necessary glassware and apparatus needed for practical

1.1 Health and Safety Equipment

Know the location and operation of the following safety equipment indicated to you during the orientation practical:

- The closest emergency exit

- The closest emergency phone and emergency phone numbers

- Closest fire alarm

- Fire extinguishers and explanation of use

- Safety showers and explanation of operation

- Eye wash facilities

- First aid kit

# PRACTICAL 2 QUANTITATIVE ANALYSIS OF FOOD COMPONENTS

## 2.1: DETERMINATION OF CRUDE PROTEIN BY KJELDAHL METHOD

Objective

The method is used for the quantitative determination of crude protein in all types of foods.

Definition

Crude protein is total nitrogen multiplied by protein factor. It is expressed in g per 100 g sample.

Total nitrogen content includes nitrogen primarily from proteins and to a lesser extent from all organic nitrogen containing non-protein substances. For practical purposes, non-protein nitrogen is assumed to be of little significance.

Principle

The method is based on the digestion of proteins and other organic food components in the sample with sulfuric acid in the presence of catalyst e.g. sodium or potassium sulfate to release nitrogen from protein and retain it as ammonium salt. Ammonia gas is liberated upon addition of excess alkali (concentrated sodium hydroxide) and is distilled into a boric acid solution to form ammonium-borate complex. The ammonia liberated from the complex is titrated with standardized hydrochloric acid. The amount of nitrogen in the sample is determined from the milligram equivalent of the acid used. Crude protein is determined by multiplying the nitrogen content with a conversion factor specific to the food matrix.

Materials

- Sulfuric acid, concentrated, A.R. grade or equivalent

- Catalysts: Potassium sulfate and or copper sulfate with or without selenium (IV) dioxide

Note: For environmental and health concerns mercury-containing catalysts should not be used.

- Sodium hydroxide solution, 50%. Weigh 500 g sodium hydroxide, dissolve in water and make up to 1 L with H2O.

Note: The reaction of NaOH with water is exothermic. Place reaction vessel in a basin of ice-cold water when dissolving it.

- Boiling chips.

- Ammonium sulfate, A.R.

- Hydrochloric acid, 0.1 N standard solution. Pipette 8.3 mL of concentrated hydrochloric acid to approximately 500 mL distilled H2O in 1 L volumetric flask soaked in ice cold water. Allow to cool and make up to volume with distilled H20. Pour the solution in 1 L brown bottle and let the solution stand for 2 or 3 days with occasional shaking before standardization.

- Standardization of HCl solution stated in 6.6.

- Weigh accurately about 0.47 g AR sodium tetraborate decahydrate (borax) into a 250mL conical flask, dissolve in about 50mL water and add a few drops of methyl red indicator solution. Titrate with the hydrochloric acid from a burette until the colour changes to pink at the endpoint. [Methyl red indicator solution: dissolve about 1g of methyl red in 600mL alcohol and dilute with 400 mL water.]

Normality = wt borax (g) x 1000

mLsHCl titrant x 190.72

- Methyl red/bromcresol green indicator solution. Dissolve 0.2 g methyl red and dilute to 100 mL with 95% ethanol. Dissolve 1.0 g bromcresol green and dilute to 500 mL with 95% ethanol. Mix one (1) part methyl red solution with five (5) parts bromcresolgree solution.

- Note: Methylene blue, 0.2% in 95% ethanol and methyl red, 0.2% in ethanol (1:2) indicator can also be used.

- Boric acid solution, 4% with indicator. Dissolve 40 g boric acid in hot distilled water, cool and make up to 1 L with distilled water and add 3 mL methyl red/bromcresol green indicator solution. Solution will be light orange in color.

Equipment

Kjeldahl and other commercial systems (such as Kjeltec/Tecator system). The system consists of three units, namely

- Digestion unit.

- Distillation unit.

- Titration unit.

- Digestion tubes.

- Erlenmeyer flask, 250 mL, 500 mL

- Magnetic stirrer and magnetic bar.

- Drying oven, capable of being controlled at 100-120°C and equipped with a suitable thermometer.

- Balance, analytical, 200 g capacity with 0.1 mg sensitivity

Cautions and Hazards

- Always wear protective devices, e.g. face mask, goggles and gloves when handling toxic and corrosive chemicals.

- Understand the toxicity and safety of the reagents used before starting the method.

- Use pipette aides in handling acids.

- Digest sample under a hood to reduce inhalation of acid fumes.

- Analysis should be done in an ammonia-free environment.

Procedure

Sample preparation

Grind or blend sample until homogenous. If sample cannot be analyzed on the same day, keep in screw-cap bottle in a freezer. For samples intended for analysis of vitamins or other labile nutrients, flush sample with nitrogen before storing.

- Blank: include two reagent blanks (containing all reagents used in nitrogen analysis except the sample) in every batch of analysis to subtract reagent nitrogen from the sample nitrogen.

- Test sample

- Thaw out sample to room temperature and mix the sample thoroughly.

- Weigh in duplicate 2–10 g sample (depending on the nitrogen content of the sample) into the digestion tube.

- Add 5-7 g catalyst and 1 glass bead to prevent solution from bumping and 10 –20 mL sulfuric acid.

- Place digestion tube in the digester. Digest mixture initially at low temperature to prevent frothing and boil briskly until the solution is clear and is free of carbon or until oxidation is complete.

Note: the digestion time and volume of sulfuric acid required depends on the material to be digested. If the digest is still yellowish, cool the digest and add an additional 5-10 mL sulfuric acid.

- Continue digestion until a clear digest is obtained.

- Heat for another hour after the liquid has become clear to complete breakdown of all organic matter.

- Place a 250-500 mL Erlenmeyer flask containing 50 Ml of 4% boric acid with indicator as receiver on the distillation unit.

Note: the tip of the condenser should extend below the surface of the acid solution

- Add 100 mL of water and 70 mL of 50% sodium hydroxide to the digests and start distillation.

Note: Make sure excess NaOH is added to neutralize sulfuric acid to ensure complete release of ammonia.

- Distill until all ammonia has been released or approximately > 150 mL distillate is obtained.

Note: Use condenser with ice cold water to effectively capture all distilled ammonia.

- Lower the receiver flask so that the delivery tube is above the liquid surface and continue the distillation for1-2 minutes.

- Finally, rinse the delivery tube with water and allow the washings to drain into the flask

- Titrate the distillate with the standardized 0.1N hydrochloric acid until the first appearance of the pink color

- Record volume of acid used to the nearest 0.05 mL.

Note: Always check the normality of the standardized 0.1NHCl.

Calculation

N (g%) = (mL 0.1N HCl sample – mL 0.1N HCl blank) x 0.0014 x N HCl x100

Weight of sample

Protein (g per 100g) = % total nitrogen x appropriate nitrogen conversion factor (see Appendix)

Report test results (in g per 100 g sample) to one decimal place.

Acceptance of test results

Accept test results if one or more of the following conditions are satisfied:

- Duplicate results should not differ by more than 5% of the mean.

- Mean concentration of duplicate results of In-house Food Reference

- Material should be within +2 SD in the control chart based on established acceptance criteria.

References

Horwitz W (2000) (editor). Official Method of Analysis of AOACInternational. 17th Edition. AOAC International, Maryland, USA.

WHO (1973). Report of a Joint FAO/WHO Ad Hoc Expert Committee on Energy and Protein Requirements, WHO Technical Report SeriesNo. 522, WHO, Geneva.

Greenfield H and Southgate DAT (1992). Food Composition Data:Production, Management and Use. Elsevier Applied Science, UK.

Kirk RS and Sawyer R (1991). Pearson’s Composition and ChemicalAnalysis of Foods, 9th Edition. Longman Scientific & Technical, Essex,England.

## 2.2: DETERMINATION OF TOTAL FAT BY MANUAL EXTRACTION

Objective

The method is used for the quantitative determination of total fat in foods using manual extraction.

Definition

Fat includes fatty acids, triglycerides, esters, long chain alcohols, hydrocarbons, other glycol esters and sterols determined by the method. It is expressed as g fat per 100g sample.

Principle

The sample is hydrolyzed by hydrochloric acid at 70-80oC. Protein, if any, can be dissolved in the acid, crude fat is then manually extracted by diethyl and petroleum ether. The solvent is removed by evaporation and the oil residue is dried and weighed.

Materials

- Petroleum ether (boiling point 35-60 OC)

- Ethyl alcohol 95 %

- 4 N Hydrochloric acid

- Diethyl ether, free from residue on evaporation.

- Petroleum ether, b.p.35-60 OC

Equipment

- Round flat bottom flask or beaker

- Thimble

- Cotton wool

- Condenser

- Extraction glassware (separating funnel or Rohring tubes or Majonnie tubes or equivalent extracting container)

- Glass funnel

- Hot air oven

- Cylinder

- Water bath

- Desiccator

- Analytical balance

- Filter paper (Whatman No. 541)

Cautious and hazards

Petroleum ether is toxic (boiling point 35-60°C), and flammable, handle with care.

Procedure

Sample preparation

- Place 2 g dried sample (W1) in a 250 mL Erlenmeyer flask or extraction tube, add 2 mL alcohol. Stir to moisten all particles (moistening of sample with alcohol prevents lumping on addition of acid).

- Add 10 mL of the diluted 4N HCl and mix well. Set the flask on the heater and reflux for 30 min. If the tube is used, place the tube in water bath held at 70–80°C and stir at frequent intervals until sample is completely hydrolyzed (usually 30–40 min).

- Add 10 mL alcohol and cool.

- If the hydrolysis has taken place in a flask, transfer the digested mixture to extraction glassware. Rinse the flask and pour into the extraction tube with 25 mL diethyl ether in three portions.

- Close the tube with cork and shake vigorously for 1 min. Add 25 Ml petroleum ether and again shake vigorously for 1 min.

- Let stand until upper liquid is practically clear.

- Transfer as much as possible of the ether-fat solution into a pre-weighed

- 125 mL flask by filtering it through a funnel containing a plug of cotton packed firmly in the stem part. Allow free passage of ether into the flask.

- Before weighing the flask, dry it in drying oven at 1005°C and then let cool in a desiccator and weigh (W2).

- Repeat extraction of the liquid sample remaining in tube twice using the same solvent. Each time, transfer the clear ether solutions through the same funnel into the same flask. When finished, rinse inside and outside of the funnel into the same flask.

- Evaporate solvents completely on a water bath at 70-80°C

- Dry fat in an oven at 100+5°C until constant weight is obtained.

- Allow the flask to cool in a desiccator and weigh (W3).

Calculation

Total Fat (g/100 g) = (W3 – W2) X 100

W1

Where: 	W1 = Weight of sample

W2 = Weight of dried flask before fat extraction

W3 = Weight of dried flask after fat extraction

Acceptance of test results

- Duplicate results should not differ by more than 5% of the mean.

- Mean concentration of duplicate results of the reference materials should be within +2 SD in the control chart based on established acceptance criteria.

References

Horwitz W (2000) (editor). Official Method of Analysis of AOAC International. 17th Edition. AOAC International, Maryland, USA, 920.39, p 33.

## 2.3: DETERMINATION OF STARCH BY ACID HYDROLYSIS METHOD

Objective

The method is used for the quantitative determination of starch in foods.

Definition

Starch constitutes the major component of the polysaccharides in most foods and consists mainly of amylose and amylopectin.

Principle

The test sample is dispersed in water and heated in acidic solution to hydrolyze the starch, releasing the sugars. The resulting sugars are then determined by titration with Fehling solution according to the method of Lane- Eynon (see Determination of Total Sugars by the Lane-Eynon Method).

Reagents

- Hydrochloric acid (HCl), 35%

- Sodium hydroxide pellet (NaOH)

- 50% Sodium hydroxide solution: dissolve 50 g of NaOH in 100 mL of distilled water.

- For other reagents, refer to the list for sugar determination by the Lane Eynon method

Apparatus

- Round bottom flask 250 mL

- Reflux condenser

- Glass beads

- Volumetric flask 250 and 500 mL

- Filter paper No. 541

- Litmus paper

- Hotplate

- For other apparatus, refer to the list for sugar determination by the

- Lane-Eynon method

Cautious and hazards

- Understand the toxicity and safety of the reagents used before starting the method.

- Use safety devices such as tong or gloves in handling hot containers.

Procedure

Preparation of sample

- Grind or blend sample until homogenous. If sample cannot be analyzed on the same day, keep in screw-cap bottle in a freezer.

Sample gelatinization and hydrolysis

- Test sample is mixed thoroughly and about 3 g is weighed (with an accuracy of 0.01 g) into a 250 mL round bottom flask. 200 Ml of distilled water is added, followed by 20 mL of concentrated HCl and 3-4 glass beads.

- Then the round bottom flask is equipped with a reflux condenser. The heat is applied until boiled and clear solution is obtained. Cool to room temperature.

- The test solution is neutralized by adding 50% NaOH (checked by litmus paper) and the test solution is filtered through Whatman No 541 into a 250 mL volumetric flask. Dilute to the mark with distilled water.

- For the analysis of sugars using Fehling solution, follow the method of Lane-Eynon.

Calculation

Calculate the starch content as follow:

- Starch, g per 100 g = % Total sugar x 0.9

- Total sugar is obtained from the method of Lane-Eynon.

- Report test results (in g per 100 g sample) to one decimal place.

Acceptance of results

Accept test results if the following condition is satisfied.

- Duplicate results should not differ by more than 10% of the mean.

- Mean concentration of duplicate results of quality control sample or certified reference material should be within +2 SD in the control chart based on established acceptance criteria.

Appendix/supplementary notes

- Methods based on acid hydrolysis also measure non-starch polysaccharide (NSP), limiting their use to situations in which a low degree of accuracy is permissible or the amount of NSP are known to be small.

- The preferred methods for the analysis of starch are based on the use of enzymatic hydrolysis coupled with a specific glucose assay of the hydrolysate.

- The AOAC (reference 3.1) also contains enzymatic hydrolysis methods for the analysis of starch in a variety of other foods, such as cereal and cereal products (32.2.05 and 32.2.05A) and in condensed or dry milk products (4.7.03).

Reference

- Horwitz W (2000) (editor). Starch in baking powders, 25.1.11, Official Method of Analysis of AOAC International. 17th Edition. AOAC International, Maryland, USA.

- Greenfield H and Southgate DAT (1992). Food Composition Data: Production, Management and Use. Elsevier Applied Science, UK.

- Kirk RS and Sawyer R (1991). Pearson’s Composition and Chemical Analysis of Foods, 9th Edition. Longman Scientific & Technical, Essex, England.

## 2.4: DETERMINATION OF INDIVIDUAL SUGARS BY HIGH PERFORMANCE LIQUID CHROMATOGRAPHY (HPLC)

Objective

The method is used for the quantitative determination of individual sugars in beverages (malt beer, soft drink, lemonade, table wine, and orange juice), honey, marmalade, foods and foods products.

Definition

Sugars determined by this method consist of mono-, di- and tri-saccharides.

Principle

- Beverages and honey are appropriately diluted with water, filtered if required, then chromatographed on HPLC column to separate the individual sugars.

- Foods and food products are extracted with aqueous ethanol. After evaporation of the ethanol, the sugars left in aqueous solution are separated by HPLC.

- Samples with high fat content should be defatted using petroleum ether before extraction step.

- For samples with high protein content, the protein should be precipitated by potassium hexacyanoferrate and zinc sulfate before extraction step.

Materials

- Calcium carbonate, AR

- Ethanol, 85% (v/v), in distilled water, AR

- Standard sugars: glucose, fructose, sucrose, maltose and lactose

- Distilled water, ultra-pure

- Ethylenediaminetetraacetic acid (EDTA), Disodium-calcium salt.

- Petroleum ether, AR

- Acetonitrile, HPLC grade

- Ethanol, HPLC grade

- Potassium hexacyanoferrate, 15 % (v/v) in distilled water

- Zinc sulfate, 30 % (v/v) in distilled water

Equipment

- High pressure liquid chromatograph system

- Hypersil (APS2) NH2, 5 µm (ThermoHypersil-Keystone) with guard column, 250 x 4.6 (id) mm or equivalent

- Analytical balance

- WhatmanTM filter paper No. 41 and 42

- Funnels

- Flat-bottom flask, 250 mL

- Ultra-filtering equipment with membrane filter 0.45 µm, diameter 47mm

- Sample Clarification Kit with cellulose acetate membrane filter 0.45um, diameter 13 mm

- Rotary evaporator

- Sample vials

- Erlenmeyer flask, 125 mL

- Volumetric flask: 25, 50, 100, 500 and 1000 mL

- Shaking water bath

- Beaker, 50 mL

- Pipette, 2, 5, 10, 20, 25 and 50 mL

- Spatula

Cautions and hazards

- Understand the toxicity and safety of the solvents used before starting the method.

- Acetonitrile is carcinogenic; wear gloves to reduce contact with skin.

Procedure

- Sample preparation procedures prior to injection into the HPLC

- Beverages such as orange juice, malt beer, soft drink, lemonade, table wine are filtered through 0.45 µm membrane.

- Honey is dissolved in hot water, approximately 1 g in 10 mL water, and filtered through 0.45 µm membrane.

- Marmalade is dissolved in hot water & insoluble parts were centrifuged off. The clear portion is filtered through 0.45 µm membrane.

- Foods and food products

  - Samples containing high fat.

Weigh accurately 1-5 g finely ground sample into 50 mLcentrifuge tube. Add 30 mL petroleum ether, vortex anddiscard petroleum ether. Repeat extraction 3 times. Drysample in oven 60oC for 1 h to remove any residualpetroleum ether. Transfer defatted sample into 125 mLErlenmeyer flask. Add 1-3 g CaCO3 to neutralize sample. Add 25 mL 85% ethanol to sample, cap flask with aluminum foil, place on shaking water bath at 60oC for 1 h. Remove sample from the shaking water bath and immediately filter the solution through a filter paper into a 250 mL flat-bottom flask. Repeat the extraction with 25 mL boiling 85% ethanol for three times. Evaporate the ethanol on rotary evaporator at 45oC until the remaining aqueous solution is approximately 3 mL. The aqueous solution is transferred by using a pipette to a 10 mL volumetric flask, made up to volume with distilled water. The solution is filtered through an ultrafilter (0.45 µm). Keep samples in a sample vial and inject into HPLC.

  - Samples containing high protein.

Weigh accurately 1-5 g finely ground sample into 25 mL volumetric flask. For solid sample, add 5 mL distilled water to disperse the sample. Pipette 1.25 mL 15%potassium hexacyanoferrate and 1.25 mL 30% zinc sulfate. Mix well and stand for 15 min. Add 5 mL acetonitrile and make to volume with distilled water. Mix well and stand overnight. The solution is filtered through an ultrafilter (0.45 µm). Keep samples in a sample via land inject into HPLC.

  - Samples containing high fat and high protein.

Weigh accurately 1-5 g finely ground sample into 50 mL centrifuge tube. Add 30 mL petroleum ether, vortex and discard petroleum ether. Repeat extraction 3 times. Dry sample in oven 60oC for 1 h to remove any residual petroleum ether. Transfer defatted sample into 25 mL volumetric flask. For solid sample, add 5 mL distilled water to disperse the sample. Pipette 1.25 mL 15%potassium hexacyanoferrate and 1.25 mL 30% zinc sulfate. Mix well and stand for 15 min. Add 5 mL acetonitrile and make to volume with distilled water. Mix well and stand overnight. The solution is filtered through an ultrafilter (0.45 m). Keep samples in a sample via land inject into HPLC.

  - Samples containing low fat and low protein.

Weigh accurately 1-5 g finely ground sample into 125mL Erlenmeyer flask. Add 1-3 g CaCO3 to neutralize sample. Add 25 mL 85% ethanol to sample, cap flask with aluminum foil, place on shaking water bath at 60oC for 1 h. Remove sample from the shaking water bath and immediately filter the solution through a filter paper into a 250 mL flat-bottom flask. Repeat the extraction 30 Main nutrients: Proximate composition, starch and sugar with 25 mL boiling 85% ethanol three times. Evaporate the ethanol on rotary evaporator at 45oC until the remaining aqueous solution is approximately 3 mL. The aqueous solution is transferred by using a pipette to a 10 mL volumetric flask, made up to volume with distilled water. The solution is filtered through an ultrafilter (0.45µm). Keep samples in a sample vials and inject into HPLC.

  - Samples containing high salt (e.g. fish sauce and seasonings).

Weigh accurately 1-5 g homogenised sample into 25 mL volumetric flask. Make up to volume with 85% ethanol. Mix well and filter through an ultrafilter (0.45 µm). Keep samples in a sample vial and inject into HPLC.

- Standard sugar preparation

- Prepare 2% of each individual sugar for determination of retention time

- Prepare 2% solution of sugar mixture for quantitative determination

- HPLC equipment and condition

Column: 	Hypersil (APS2) NH2, 5 µm (ThermoHypersil-Keystone) with guard column, 250 x 4.6 (id) mm or equivalent

Mobile phase: Acetonitrile: distilled water: ethanol = 82: 17.5: 0.5 Detector: 	RI detector

Conditions: Flow rate 1.5 mL/min Injector volume 20 µL

Column temperature 25-30 oC

RI detector temperature 25-30oC

- Determination

- Inject sugar standards and sample solution (10 –20 µL), into column with appropriate conditions as indicate above.

- Measure areas or peak heights of each sugar peak in sample and standard.

Calculation

Amount of each sugar (g/100g) = ASPL X CSTD  x   V

Astd		W

Where: 	Asp = area/peak height of each sugar in sample solution

Astd = area/peak height of sugar standard

Cstd = concentration of sugar standard (g/100 mL)

V = total volume of prepared sample solution (mL)

W = weight of sample (g)

Results are reported to the nearest 0.1 g/100 g

Acceptance of results

Accept test results if the following condition is satisfied.

- Duplicate results should not differ more than 10% of the mean.

- Mean concentration of duplicate results of quality control sample or certified reference material should be within +2 SD in the control chart based on established acceptance criteria.

References

AOAC International (1993). Methods of Analysis for Nutrition Labelling. Chapter 33. Sugars (Mono & Di), Glucose, Fructose, Sucrose and Maltose in Presweetened Cereals Liquid Chromatographic Method (982.14); Sugars (Mono & Di), Separation of Sugars in Honey Liquid Chromatographic Method (977.20).

Will RBH and Greenfield H (1984). Laboratory instruction manual for food composition studies. Department of Food Science and Technology, the University of New South Wales, p 59.

Waters (1987). Choosing the Right Column Chemistry for carbohydrate Analysis, Notes Food & Beverage, Waters Chromatography Division Millipore Corporation, 2:4-6.

## 2.5: DETERMINATION OF ASH BY GRAVIMETRIC METHOD

Objective

The method is used for the quantitative determination of ash in raw, cooked and processed foods.

Definition

Ash content refers to the total mineral residue left after incineration of organic matter. It has no nutritional significance per se, but the value for ash is a useful check in summing up the proximate composition of food and a measure of its mineral content. It is expressed as g ash per 100 g sample.

Principle

The method involves oxidation of all organic matter by incineration in a furnace at a specified temperature (<550°C). Ashing above 650°C will volatilize inorganic salts like alkali chloride and a portion of ash will fuse and enclose some carbon, preventing them from being ignited. The residue left after incineration is the ash content of the sample.

Materials

N Nitric acid or dilute HCl (1:2.5)

Apparatus

- Furnace, capable of being controlled at 100° to 600°C

- Air oven

- Hotplate or Bunsen burner or electric coil

- Analytical balance, 200 g capacity with 0.1 mg sensitivity

- Desiccator with desiccant such as silica gel. Ensure that the desiccants activated prior to use by heating in an oven at 100°C until blue.

- Boiling water bath with removable rings to hold the containers.

- Tongs

- Porcelain crucible or silica dish

- Spatula

Cautious and hazards

- Always use tongs and gloves in handling crucibles.

- Wear face/eye protection when opening hot furnace.

Procedure

- Heat marked crucible in a furnace at 500 - 550ºC for 2 - 3 h.

- Lower the furnace temperature to 180ºC and transfer the crucibles into a desiccator, cool for 30 min and weigh (W1).

- Weigh sample in duplicate into the pre-weighed crucible dish (W2), 2 – 4 g for dry samples and 10 g for wet samples.

- For dry samples, char over a hotplate, initially at low temperature to avoid spattering. Increase the temperature gradually until smoking ceases.

- For wet or liquid samples, pre-dry over a boiling water bath until samples are dry. Char the sample as in 8.4.

- Incinerate the charred samples in a furnace at 500-550ºC until the residue is uniformly white or nearly white.

- If sample is not completely white, moist ash with a few drops of water ordiluted acid. Evaporate on water bath and repeat heating in the muffle furnace for 30 – 60 min until constant weight is obtained.

NOTE: If ash samples are to be used for mineral analysis, temperature of the furnace should not exceed 450OC. too high a temperature may cause the volatilization of certain elements particularly Fe, K, Na, S, Cl & P. It may also cause the mineral matter to fuse and melt.

- Decrease the temperature of the furnace to 180ºC and transfer the crucibles into a desiccator, cool for 30 min and weigh (W3).

Calculations

Ash, g per 100 g =(W3- W1) x 100

(W2 – W1)

Where: 	W1 = weight of crucible

W2 = weight of crucible + sample

W3 = weight of crucible + ash

Report test results (in g per 100 g sample) to one decimal place.

Acceptance of results

Duplicate results should not differ by more than 5% of the mean.

References

Horwitz W (2000) (editor). Official Method of Analysis of AOAC International. 17th Edition. AOAC International, Maryland, USA.

Greenfield H and Southgate DAT (1992). Food Composition Data: Production, Management and Use. Elsevier Applied Science, UK.

Kirk RS and Sawyer R (1991). Pearson’s Composition and Chemical Analysis of Foods, 9th Edition. Longman Scientific & Technical, Essex, England.

# PRACTICAL 3 # DETERMINATION OF CHEMICAL PROPERTIES OF FOODS

## 3.1: DETERMINATION OF pH and TITRATABLE ACIDITY

Objective

Determine the titratable acidity and pH of food samples

Principle

The volume of a standard base used to titrate the organic acids in foods to a phenolphthalein endpoint can be used to determine the titratable acidity.

Materials

- Apple juice, 60 ml

- 3 Beakers, 250 ml

- 2 Burets, 25 or 50 ml

- 4 Erlenmeyer flasks, 250 ml

- Funnel, small, to fit top of 25 or 50 ml burette

- Graduated cylinder, 50 ml

- Soda, clear, 80 ml

- 2 Volumetric pipettes, 10 or 20 ml

Equipment

- Hot plate

- pH meter

Cautions and hazards

- Adhere to normal laboratory safety procedures.

- Wear safety glasses at all times.

- Waste likely may be put down the drain using a water rinse, but follow good laboratory practices outlined by environmental health and safety protocols at your institution.

Procedure

- Soda

- Do at least duplicate determinations for unboiled soda and for boiled soda sample; open soda well before use to allow escape of carbon dioxide so that the sample can be pipetted.

- Unboiled soda: Pipette 20 ml of soda into a 250- ml Erlenmeyer flask. Add ca. 50 ml CO2-free dd water. Add 3 drops of a 1% phenolphthalein solution and titrate with standardized NaOH (ca. 0.1 N) to a faint pink color (NaOH in burette, from Method A). Record the beginning and ending volumes on the burette to determine the total volume of NaOH solution used in each titration. Observe the endpoint. Note whether the color fades.

- Boiled soda: Pipette 20 ml of soda into a 250-ml Erlenmeyer flask. Bring the sample to boiling on a hot plate, swirling the flask often. Boil the sample only 30–60 s. Cool to room temperature. Add ca. 50 ml CO2-free dd water. Add 3 drops of the phenolphthalein solution and titrate as described above. Record the beginning and ending volumes on the burette to determine the total volume of NaOH solution used in each titration. Observe the endpoint. Note whether the color fades.

- Apple Juice

- Standardize the pH meter with pH 7.0 and 4.0 buffers, using instructions for the pH meter available.

- Prepare as described below three apple juice samples that will be compared:

A – Apple juice (Set aside, to recall original color)

B – Apple juice; titrate with std. NaOH

C – Apple juice; add phenolphthalein; titrate with std. NaOH; follow pH during titration.

Procedure

- Into each of three (A, B, C) 250-ml beakers, pipette 20 ml of apple juice. Add ca. 50 ml CO2-free water to each. To beaker C, add 3 drops of a 1% phenolphthalein solution.

- Using two burets filled with the standardized NaOH solution (ca. 0.1 N), titrate Samples B and C simultaneously. Follow the pH during titration of Sample C containing phenolphthalein.

(Note: If only one burette is available, titrate Samples B and C sequentially, i.e., add 1 ml to B, then 1 ml to C.)

- Record the initial pH and the pH at ca. 1.0 ml intervals until a pH of 9.0 is reached. Also, observe any color changes that occur during the titration to determine when the phenolphthalein endpoint is reached. Sample A is intended to help you remember the original color of the apple juice. Sample B (without phenolphthalein) does not need to be followed with the pH meter, but is to be titrated along with the other beaker to aid in observing color changes.

Data and calculations

- Soda

  - Using the volume of NaOH used, calculate the titratable acidity (TA) of each soda sample as percentage citric acid, then calculate the mean TA of each type of sample (MW citric acid = 192.14; equivalent weight = 64.04) (Note: For equation, see Chap. 13 of Nielsen, Food Analysis Book)

  - Sample calculation:

%Acid= (ml base titrant) x (N of base in mol / liter) x (Eq. Wt. of acid)

(Sample volume in ml) x 10

N NaOH = 0.1019 N

ml base = 7 ml

Eq. wt. citric acid = 64.04

Vol. sample = 20 ml

%Acid = (7 x0.1019 x64.04)

= (20x 10)

= 0.276%citric acid

- Apple juice

Sample B

  - Color change during titration:

  - Color at end of titration:

Sample C (titrate to >pH 9.0

ml NaOH 	1 	2 	3 	4 	5	 6 	7 	8 	9 	10

pH

ml NaOH 	11 	12 	13 	14 	15	1 6 	17 	18 	19 	20

pH

Plot pH versus ml of 0.1 N NaOH (but use the normality of your own NaOH solution) (pH on the y-axis) for the sample that contained phenolphthalein (Beaker C). Interpolate to find the volume of titrant at pH 8.2 (the phenolphthalein endpoint).

Calculate the titratable acidity of the apple juice as percentage malic acid (MW malic acid = 134.09; equivalent weight = 67.04).

## 3.2: DETERMINATION OF ASCORBIC ACID OR VITAMIN C

Objective

To determine the amount of Vitamin C or Ascorbic acid that give rise to dehydro-ascorbic acid due to oxidation

Definition

Vitamin C or Ascorbic acid is an enediol isomer of 2-keto-l-glucanolactone with a configuration similar to that of L-glucose. Oxidation of ascorbic acid gives rise to dehydro-ascorbic acid and both forms are physiologically active.

Principle

Titrimetric estimation of Vitamin C is conventionally done using 2,6-dichlorophenol indophenol dye solution.

This dye is blue in alkaline solution and red in acidic solution. Ascorbic acid reduces the dye to a colorless form. Reaction is quantitative and specific for ascorbic acid at pH 1.0 - 3.5.

Materials

- 4% Oxalic acid: 40 g of oxalic acid is dissolved in 100ml of distilled water (w/v)

- DCPIP dye solution: Dissolve 0.250 g of sodium salt of 2,6-dichlorophenol indophenols in about 500 ml of water containing 0.210 g of NaHCO3 and dilute to 1 liter of water. Store the solution in refrigerator and standardize it with freshly prepared standard solution of ascorbic acid every time just before use.

- Standard ascorbic acid (C6H8O6): 0.01% Ascorbic Acid dissolved in oxalic acid.

Procedure

- Take 5 g of fruit sample (filtered juice) make up to 100 ml with 4% oxalic acid.

- Take 5 ml of the sample from the 100 ml and add 10 ml 4% oxalic acid and titrate against the dye 2,6-dichlorophenol indophenol

- The end point is determined by the appearance of pink color which should persist for at least 15 seconds.

- Standardization of the dye solution: The dye solution is needed to be standardized simultaneously. From this 5 ml of standard ascorbic acid solution is taken and to this, 10 ml of 4% oxalic acid is added. Mixed well and titrated against DCPIP.

Result

Titer value (TV) is noted and this will be in a range between 0.1 – 1.0 ml

Calculation

0.5 mg/V1 ml (3.3) x V2 (T.V)/5 ml/Wt. of sample (5 g) x 100

60.6 x T.V =……………mg/100g

Reference

Rashida Rajuva TA & Joy PP. 2014. A Food Technology Lab Manual. Pineapple Research Station (Kerala Agricultural University), Vazhakulam-686 670, Muvattupuzha, Ernakulam, Kerala. Web: www.kau.edu/prsvkm, http://prsvkm.tripod.com

## 3.3: DETERMINATION OF TOTAL SOLUBLE SOLIDS

Objective

To determine the amount of sugar present in the sample

Definition

Total soluble solids (TSS) of a given sample of fruit juice representing various chemical substances present in it in soluble form. It indicates a measure of sugars present in the sample. The amount of TSS present in the juice of a fruit is also considered to be a reliable index in judging its maturity. In accordance with the harvest – maturity of many fruits is assessed in considering the TSS of their juices.

Principle

The TSS of a given fruit juice sample is determined in a quicker way with the help of the refractometer. The instrument works on the principle of refractive index of the sample and gives the refractive index as ◦Brix.

Materials

A hand Refractometer, a dropper or a glass rod, blotting paper, absorbent cotton, rectified sprit, distilled water.

Procedure

- The lid that is covering plate of the refractometer which rest over the prism-plate and is attached with it at the base end with a hinge which can be unfolded backward. By doing so, both the lid and the prism-plate are exposed.

- The lid and the prism-plate are then carefully washed with a jet of clean water to ensure that they have no stain on its surface.

- Water adhered on the prism plate and the lid as well as the surrounding parts of them, if any is completely wiped off with blotting paper or absorbent cotton.

- The lid of the prism plate is then washed with distilled water and the water adhered on them is blotted out. The cleaning is best done by rubbing the lid and the prism plate gently and carefully with absorbent cotton, soaked with rectified spirit.

- Then, with the help of a previously cleaned dropper or a glass rod, a drop of distilled water is carefully dropped on the surface of the prism plate. The lid is folded forward and placed over the prism plate to cover it. At this position, the lid and the prism plate are firmly held together with fingers to avoid unfolding of the lid.

- The refractometer is held to point towards light. The eyepiece of it is brought close to any eye of the observer who should look into the eye-piece to have a view of the image of the scale. The scale focusing knob should be conveniently rotated to adjust it at such a position where the scale is most clearly visible. The shaded part would be seen to insert the unshaded part at zero position of the scale which indicates no reading with respect to distilled water. If it is not there then the reading should be brought to zero by rotating the scale calibrating screw.

- The lid is then folded backward. The distilled water that remains adhered over the lid and the prism-plate is completely blotted out and dried in air for a few minutes.

- A clear sample of fruit juice, TSS of which is to be determined is taken in the dropper, or a drop may be taken with the glass rod. A drop of juice is then carefully placed on the prism plate.

- Reading of the juice sample as oBrix is obtained and amount of TSS is expressed accordingly.

Reference

Rashida Rajuva TA & Joy PP. 2014. A Food Technology Lab Manual. Pineapple Research Station (Kerala Agricultural University), Vazhakulam-686 670, Muvattupuzha, Ernakulam, Kerala. Web: www.kau.edu/prsvkm, http://prsvkm.tripod.com.

# PRACTICAL REPORT GUIDELINES

Report writing

All practical reports should be written in the format of a research article as prescribed by the international Journal of Food Science and Technology. A few basic guidelines are given below:

Title

The title must be as concise and descriptive as possible. E.g. If you conducted a practical on the determination of moisture content in apples, maize and barley, it is no use giving the title “Moisture content”. Instead, provide a little that fully describes your report i.e. “The determination of the moisture content of apples, maize and barley”. An effective title will encourage a reader to go ahead and read the rest of the document.

Abstract

This is very concise statement which should address the following elements of your report:

- Scope of your experiment (i.e. what is the problem?);

- Objectives of your study (i.e. what do you hope to achieve in your study?);

- Methodology used (i.e. how did you go about your study?);

- Summary of important results (i.e. what have you found?);

- Main conclusions.

This section is always written in past tenses as you are reporting something (your experiment) that is historical.  There should be no references made to literature in this section, and lengthy background information should be avoided.  When addressing the methodology used, experimental details are not necessary, and only important/notable results should be reported (e.g. trends, ranges, patterns etc).  The abstract should be no more than 200 words.

Hint: Aim to write 1 or 2 sentences covering each of the elements mentioned.

Introduction

The purpose of this section is to provide the reader with sufficient background information on your study to enable the reader to understand your results easily.  This section also makes it possible for the reader to evaluate your results without needing to refer to other publications.  The introduction provides motivation for your study and should briefly and clearly state why you are writing the report (usually a short sentence at the end of this section).  References must be chosen very carefully to ensure that the most relevant background to the study is given.  The introduction should encompass the following:

- The nature of the problem under investigation (Broad background to the problem);

- Review of relevant literature (fundamental concepts, other authors’ findings etc);

- Clear statement of hypothesis;

- Aim/motivation of the study.

Often authors tend to struggle to find the right balance between giving too much information, and not giving enough.  One should avoid giving too much irrelevant (i.e. nonscientific) background information.  It is important to give an interpretation of other workers data with relevance to your study and not simply state everything they did in their study.

Hint: Structure your introduction in al logical way that leads the reader towards the relevance of your study.  Move from the general to the specific as your study will look at more specific things.

Materials and methods

This section provides a very detailed summary of how you went about conducting your study.  It should provide sufficient detail to enable a reader to repeat the experiment by referring to your text.  This section should not simply be a regurgitation of the protocols set out in the manual.

You should be able to make use of the summarized protocols to construct good descriptive paragraphs of what you did.

For the materials, include any technical specifications, quantities and source of method of preparation.  Make sure you get all this pertinent information before you leave the laboratory and ask the instructor or practical assistants for help where needed.  It makes logical senses to lay out your methods in a logical chronological manner (i.e. move from sample preparation to the last treatment/analysis).

Always ensure that you include major details e.g. number of replicates performed, name of computer software used (for analysis), correct units in the correct format etc.  Do not include any justification, results or discussion in this section.  If there are several methods being presented, it is sometimes helpful to organize these under subheadings.

Hints: Make sure that it is clear and succinct so that it is easy to follow.  Do not give background information on methods used.  Do not mention any results in this section.  Do not discuss sources of error in this section.

Results

Presentation of results should also be done in a logical way (i.e. arranged in an order which is easy to follow and relate results if necessary).  Use your discretion to establish what the most effective way to present your results will be (often, graphs and figures are easier and quicker to follow than tables).  Where replicates have been used, provide an average and a standard deviation in parentheses (or ± x) to show your experimental precision (Do Not Give Raw Data in Your Report).

Figures and tables should be clearly numbered and have good descriptive titles of the data they contain.  Correct units should be indicated in parentheses next to column/row headings as well as next to axes labels on graphs.

Apart from presenting your results in tables and/or figures, you should also provide a brief informative caption explaining what each graph shows.  In case, you do not need to state every figure of data point, but simply highlight major observations in the data “(e.g. “an increase in x caused a decrease in y”).  Avoid statements like “table x shows…”

If your results are calculated from a formula, only present the final result and shows the formula in the materials and methods section.

Given below are examples of a bad and a good table:

Free amino nitrogen values

| No enzyme | 25.3 |
| --- | --- |
| Bioprotease P. Conc | 72.12 |
| Flavourzyme | 52.235 |
| Neutrase | 32.302 |
| Papain | 50.2 |
| NS 26001 | 47.59 |
| NS 26009 | 49.8 |
| NS 26068 | 30.7 |
| NS 26023 | 24.58 |

Effect of different exogenous proteolytic enzymes of free amino nitrogen (FAN) levels of sorghum grain meal mashed at 55oC for 45 min.

| Enzyme treatment | FAN (mg/100g sorghum) | % increase in FAN |
| --- | --- | --- |
| No enzyme treatment | 25.3 (3.4) |  |
| Bioprotease P. Conc | 72.1 (0.8) | 185 |
| Flavourzyme | 52.2 (3.0) | 106.3 |
| Neutrase | 32.3 (1.3) | 27.7 |
| Papain | 50.2 (0.4) | 98.4 |
| NS 26001 | 47.6 (2.0) | 88.1 |
| NS 26009 | 49.8 (0.8) | 96.8 |
| NS 26068 | 30.7 (2.5) | 21.3 |
| NS 26023 | 24.58 (0.4) | -2.8 |

All results are means of 2 replicate experiments.  Figures in parentheses indicate standard deviations.

Hints: it is also common to find that the results and the discussion section are combined.  In this case it is helpful to discuss a particular result immediately after it is mentioned in the text.  Tables are not the most effective method to use when describing relationship between variables”

Discussion

This is often the most difficult section of the paper to write as it requires an extra amount of “scientific” thorough from the author explains the “causes” of the observed “effects” (i.e. results).  Through explanations backed by evidence are necessary.  Often this evidence can be sourced from the references used in the introduction (this is only possible if the references used in the introduction are relevant).  The results should be discussed in the content of the hypothesis and literature presented in the introduction.

Generally, the discussion should include the following elements:

- Principles, relationships and generalizations shown by the results;

- Identification and explanation of “deviant” results accompanied by recommendations if these are a result of experimental error;

- Comparison of results and interpretations with previously published work giving possible reasons where experimental results don’ agree with those of other researches.

- Discussion of theoretical and practical applications of other researchers.

The discussion should always be taken in the context of the hypothesis stated in the introduction.  Only results that are included in the results section should be discussed.

Conclusion

This is where the author takes a look at the results ad discussion and sees whether they have addressed their hypothesis.  If the hypothesis has not been supported by the results, recommendations need to be made to adapt extend the hypothesis.  This section is usually very brief.

References

The correct referencing style (i.e. Harvard) should be used within the text and in the references section at the end of the paper.

General

- A report is a historical account of what you have already done.  This means that everything is written in the past tense (with the exception of the recommendations and certain exceptional cases).

- Avoid writing in the first person (i.e. I, we, us).

- Scientific writing is simple and straight to the point.  Avoid writing long winded sentences to avoid confusing the reader.

- Always make use of the spell check function and read through your report to ensure it is easily understandable.

Prescribed report format

The report writing guidelines provided in the previous serve the purpose of assisting you in the preparation of a scientifically sound report. You will be required to adhere to the following prescribed format when submitting your report for assessment:

- The overall font of the document should be in an easy to read font (e.g. Calibri, Arial, Times New Roman).

- Use a font size of 11 or 12 with 1.5 line spacing throughout the main body of the report

- The entire document should be fully justified (unless otherwise stated)

- Each page should be numbered on the bottom right.

- Use 2 cm margins.

- The title should be positioned at the top first page of your report.  It should have a font size of 1, be bold and centered.

- Your initials and surname should follow on the line directly below your initials and surname.  This should all have a font size of 12 with 1.5 line spacing and should be centered.

- The main body of the text follows the name and address and should be divided into the sections stipulated in the report writing guidelines.  Each section should be clearly distinguished from other sections by using appropriate section headings.  Section headings should be fully justified, bold and have a font size of 14.  If any sections are further divided into sub sections, these should be distinguished by using appropriate sub headings.  Sub headings should be fully justified, bold and have a font size of 12.

- Table and figure captions should have the word “table/figure” in bold and the description of the data follows thereafter in normal font size text (i.e. table 1 effect of storage temperature on ……..)

- References should be given in the specified format.

- A minimum of five (5) references are required per report; three (3) of which must be journal articles.

- A completed assessment form (available on Web Studies) should be attached as the cover page to the report.

- Reports must be submitted through Turnitin before submission.  Reports with originality score of 25% and higher, will not be marked.

# FORMAT OF REFERENCES IN REFERENCES LIST

1. National laws and Culture collections

Example in text = (Anon, 1997)

Anonymous (1985). American Type Culture Collection Catalogue of Bacteria, Phages and rDNA Vectors, 16th ed. (edited by Gherna, R., Nierman, W. &Pienta, P,). Pp. 129-130. American Type Culture Collection, Maryland.

Anonymous (1986a). The lactococci. In: National Collection of food Bacteria Catalogue of Strains, 3rd ed. Pp. 164-166. AFRC Institute of Food Research Reading.

Anonymous (1997). Food stuffs, Cosmetics and Disinfectant Act and Regulations. Act no. 54 of 1972 G.N.R. 1555/1997. Johannesburg, South Africa: Lex Partia Publishers.

2. Books, Chapters in books

Example in text = (Belitz&Grosch, 1999)

Beerens, H,.Neut, C. &Romond, C. (1986). Important properties in the differentiation of Gram-positive non-sporing rods in the general Propionibacterium, Eubacteriaum, Actinomyces and Bifdobacterium. In: Anaerobic Bacteria in Habitats other than Man (edited by E.M. Barnes & G.C. Mead). Pp 42-65. London: Blackwell Scientific Publications

Belits, H.D. &Grosch, E. (1999). Food Chemistry. Pp. 769-773. New York: Springer-Verlag Berlin Heidelberg.

Bitton, G. (1994). Wastewater Microbiology. Pp. 139-247, 371-381. New York: John Wiley & Sons, Inc.

Bushel, M.E. (1993). Application of the principles of food microbiology to biotechnology.  In: Principles of Biotechnology (edited by A. Wiseman). P. 24. New York: Surrey University Press.

Moore, W.E.C. &Holdeman, L.V. (1974). Genus 1. Propionibacterium Orla-Jensen 1909. In: Bergey’s Manual of Determinative Bacteriology, 8th ed. (edited by R.E. Buchnan&N.E. Gibbson). Pp. 933-641. Baltimore: Williams &Wikins.

Salunkhe, D.K., Bolin, H.R & Reddy (1991). Storage, processing and nutritional quality of fruits and vegetables. In: Processed Fruits and Vegetables, Volume II, 2nd ed. Pp. 195-01.  Boston: CRC Press.

Von Elbe, J.H. &Schwarts, S.J. (1996) Colourants. In: Food Chemistry (edited by O. R. Fennema). Pp. 684-693. New York, US: Marcel Dekker, Inc.

3. Theses

Example in text = (Hansmann, 1997)

Britz, T.J. (1978). A reappraisal of the taxonomy of the genus Propionibacterium Orla-Jensen 1909.  DSc (Agric) Thesis, University of Pretoria, South Africa.’

Hansmann, C.F. (1997). Dehydration of fruit withoutSulphur dioxide.  PhD in Food Science Thesis, University of Stellenbosch, South Africa.

Rattanasomboon, N. (2000). A study of the Growth and thermal resistance of the food spoilage bacterium, brochothrixthermosphacta. PhD Thesis.  The University of Birmingham, England.

4. Conference proceedings and Technical reports

Example in text = (DAWN, 1985)

AUSTERMAN-Haun, U. Seyfried, C.F. &Resonwinkel, K. H. (1997). UASB-reactor in the fruit juice industry. In: Proceedings of the 8th International Conference on Anaerobic Digestion, Sendai, Japan, Vol.1. Pp 413-420.

Blakey, N.C & Maris P.J (1987). Production of Kefir. In: Proceedings of the Symposium on Food Fermentation Process. Pp. 1-15. September 1987. Cargriali, Sardinia.

Crawford, G.V. &Teletzka, G.H. (1993). Performance of a hybrid anaerobic process in treatment food waste streams In: Proceedings of the 41st industrial Waste Conference, Pp. 196-203. Purdue University, West Lafayette, Canada.

DAWS (Department of Agriculture and Water Supply) (19850. Guidelines for the disposal of effluent on land in South Africa.  Information Bulletin No. B1/1. Government Printer, Pretoria.

Dried Fruit Board.  (1992). Fifty Fourth Annual Report.  South African Fruit Board. p. 33. Wellington: Visser Press Corporation.

Earnshaw, R., Mitchell, A.A. & Banks, J.G. (1989). The use of microbial antagonism to increase the safety of chilled foods.  Technical Memorandum No. 544. Chipping Camden, The Campden Food and Drink Research Association, UK.

Ministry of Agriculture, Fisheries and Food. (1987a). Survey of Consumer Attitudes to Food Additives, Volume I, p. 45. Her Majesty’s Stationery Office, London.

Ministry of Agriculture, Fisheries and Food (1987b). Survey of Consumer Attitudes to Food Additives, Volume II, p. 133. He Majesty’s Stationery Office, London.

Suihkkok, M-L. (1989). VTT Collection of Food Microorganisms 2nd ed. Research Notes No. 1035, p. 124. Espoo, Technical Research Centre of Finland, Finland.

5. As Cited By

Example in text = (Kluyver&Donker, 1925)

Kluyber, A.J. &Donker, H.J.L (1925). PhD. Thesis, Delft (as cited by Van Niel, 1928.

Oral-Jensen, S. (1989). Studienober die Lochbildung in den emmanthaler Kasen. Zentralblatt fur Bakteriologie, Parasitenkunde, Infektionskrankheitenund Hygiene, Abt. II 4, 217,265 and 325 (as cited by Van Niel, 1928).

Rippel, I. (1927). Vorlesungenubertheoretischespeisechemie. Springer, Berlin (As cited by Janoschek. 1944.

Samain, P.P.P. (1983.  Metabolism des composis intermediaries de la fermentation methaniqe.  Thise e Doctorat, Universit, des Sciences et Techniques de Lille, France (As cited by Beerens et al, 1986)

Please Note:

If the original article is uncountable, the source from which it was cited, should also be included in the reference list.  (In other words, for the examples mentioned above, the following should then also be included the reference list: Van Niel, 1928; Janoschek, 1944 and Beerens et al., 1986.

6. Personal communication

Personal communication is only given in text and should NOT be listed in the Reference list.

Example in text = (Siebrits, J., 1997, Director, SA Preserving Company, Tulbagn, South Africa, personal communication, 17, July)

7. Articles

Example in text = 9Rastogi &Niranjan, 1998)

Birtz, T.L (1975).  The incidence of propionibacterial in South African dairy products.  South African Journal of dairy Technology, 7, 157-161.

Charfreitag, O., Collins, M.D. &Stackebrandt, E. (1988). Reclassification of Arachniapropionica as Propionibacteriumpropionicus comb. Nov. International Journal of Systematic Bacteriology, 38, 354-357.

Rastogi, N.K. &Niranjan, K. (1998). Enhanced mass transfer during osmotic dehydration of high pressure treated pie apple. Journal of Food Science. 63, 508-511.

8. Internet References

Example in text = (Foulke, 1993)

Anonymous (2001). Kefir. National Food Safety Database.  (Interned document). URL http://.food-safety.rog/el/il105.htm. 5/3/2001

Foulke, F.E. (1993). Urethane in alcoholic beverages under investigation. FDA Consumer. (Internet document). URL http://vm.cfsan.fda.gov/frf/fc0293ur/htm. 19/08/1999.

Public Health Laboratory Service (PHLS) (1999). Food poisoning corrected notifications: England and Wales, 1982-98 (Internet document). URL http://www.phls.co.uk/facts/foodtl.htm. 13/09/2002.

Please note: Always keep a hard copy of an internet document used as a reference, since a copy of it might be requested by the publisher.

9. Newspaper articles

Example in text = (Anon., 1997)

Anonymous (1997). City could run out of water by 2020. Cape Times, May 1, 1997. Vrnnerm w., (2000).  VerslagoorKaapseleweskok.  Die Burger, September 29, 2000. Viall, J. (1999).  New law ‘will milk small dairies dry’.  Cape argus, October 19, 1999.

10. Pamphlets, standards and Newsletters

Example in text = (ISO 5492, 1992)

AACC. (1995). Approved Methods of the AACC. Moisture (method 44-15). St. Paul, Minnesota: American Association of Cereal Chemists.

AOAC (1990). Official Methods of Analysis (edited by K. Helrich). Arlington, VA: Association of Official Analytical Chemists).

Saloff-Coste, C.J. (1996). Kefir. Nutritional and health benefits of yoghurt and fermented milks. Danone World Newsletter, 11, 1-7.

11. Encyclopedia

Example in text = (Marshall, 1993)

Chudoba, J. (1989). Activated Sludge-Bulking Control. In: Encyclopedia of Environmental Control Technology 9edited by P.N>Cheremisnoff).  Vol. 3. Pp. 171-202.  Houston: Gulf Publishing Company.

Marshall. V.M. (1993). Kefir. In: Encyclopedia of Food Science and Technology (edited by Y.H.Hui). Vol 3. Pp. 1804-1808.  Chichester, UK: John Wiley & Sons.

# PRACTICAL REPORT MARKING SCHEME

| PRACTICAL REPORT MARKS ALLOCATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| SECTION DESCRIPTION | MARKS |  |  |  |
| TITLE | MAXIMUM |  | AWARDED |  |
| Brief description title of experiment |  |  |  |  |
| ABSTRACT |  |  |  |  |
| Purpose of report stated |  |  |  |  |
| General experimental procedure stated |  |  |  |  |
| Basic findings stated |  |  |  |  |
| INTRODUCTION |  |  |  |  |
| Relevance of background literature |  |  |  |  |
| Summarizing of pertinent facts |  |  |  |  |
| Objectives and hypothesis clearly stated |  |  |  |  |
| Literature correctly referenced |  |  |  |  |
| MATERIALS AND METHODS |  |  |  |  |
| Sufficient information given to enable reproduction of experiment |  |  |  |  |
| Correct units of quantities consistently used |  |  |  |  |
| Number of replicates performed stated |  |  |  |  |
| RESULTS |  |  |  |  |
| Tables and figures have numbered and sufficiently descriptive captions |  |  |  |  |
| Graph axes and /or table column headings are labelled and have correct units |  |  |  |  |
| Tables and/or figures are presented in the correct format |  |  |  |  |
| Reasonable level of accuracy and precision shown |  |  |  |  |
| Good concise description of results |  |  |  |  |
| DISCUSSION |  |  |  |  |
| Good interpretation of results (trends, possible causes) |  |  |  |  |
| Good comparison of experimental data to literature with reasonable explanations |  |  |  |  |
| Discussion of theoretical and practical applications |  |  |  |  |
| Literature correctly referenced |  |  |  |  |
| CONCLUSION |  |  |  |  |
| Concise statement of major findings |  |  |  |  |
| Recommendations |  |  |  |  |
| REFERENCES |  |  |  |  |
| Correct style of referencing used |  |  |  |  |
| All references appearing in text listed |  |  |  |  |
| GENERAL |  |  |  |  |
| Report follows logical flow in all sections |  |  |  |  |
| Consistent use a clear scientific style of writing |  |  |  |  |
| Good spelling and grammar |  |  |  |  |
| TOTAL | 100 |  |  |  |